LAB

Thực hành các kết nối cơ bản

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| S1 | VLAN 1 | 192.168.1.253 | 255.255.255.0 |
| S2 | VLAN 1 | 192.168.1.254 | 255.255.255.0 |
| PC1 | NIC | 192.168.1.1 | 255.255.255.0 |
| PC2 | NIC | 192.168.1.2 | 255.255.255.0 |

Objectives

Part 1: Perform a Basic Configuration on S1 and S2

Part 2: Configure the PCs

Part 3: Configure the Switch Management Interface

Background

Trong bài Lab này, bạn sẽ thực hiện các cấu hình switch cơ bản. Sau đó, bạn sẽ thực hiện kết nối cơ bản bằng cách cấu hình địa chỉ IP trên các thiết bị switch và PC. Khi cấu hình địa chỉ IP hoàn tất, bạn sẽ sử dụng các lệnh show để xác định cấu hình và sử dụng lệnh ping để xác định kết nối cơ bản giữa các thiết bị.

Part 1: Perform a Basic Configuration on S1 and S2

Step 1: Configure S1 with a hostname.

a. 	Click S1, and then click the CLI tab.

b. 	Enter the privileged EXEC mode. Then enter the global configuration mode.

Switch> enable

Switch# configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

c. 	Configure the hostname as S1.

Switch(config)# hostname S1

S1(config)#

Step 2: Configure the console and privileged EXEC mode passwords.

a. 	Configure cisco as the console password and enable login.

S1(config)# line console 0

S1(config-line)# password cisco

S1(config-line)# login

S1(config-line)# exit

b. 	Use class for the encrypted privileged EXEC mode password.

S1(config)# enable secret class

Step 3: Verify the password configurations for S1.

a. 	To verify that the passwords were configured correctly, enter end to exit the global configuration mode. Type exit to exit the privileged EXEC mode.

S1(config)# end

S1#

%SYS-5-CONFIG_I: Configured from console by console

S1# exit

b. 	Press Enter and you will be prompted for a password to enter the user EXEC mode.

What password did you use? ____________________________________________

c. 	Type enable to enter the privileged EXEC mode. Enter the password when prompted.

What password did you use? ____________________________________________

d. 	Enter configure terminal to enter global configuration mode.

Step 4: Configure a message of the day (MOTD) banner.

In this step, you will configure a message of the day banner to warn unauthorized access. The following text is an example:

Authorized access only. Violators will be prosecuted to the full extent of the law.

Use the banner motd command with the sample message. You can choose another message.

S1(config)# banner motd "Authorized access only. Violators will be prosecuted to the full extent of the law."

Step 5: Save the configuration file to NVRAM.

a. 	Exit to privileged EXEC mode.

S1(config)# exit

S1#

%SYS-5-CONFIG_I: Configured from console by console

S1#

b. 	Enter the copy running-config startup-config command to save the configuration.

S1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Step 6: Repeat Steps 1 through 5 on S2.

Part 2: Configure the PCs

Step 1: Configure both PCs with IP addresses.

a. 	Click PC1, and then click the Desktop tab.

b. 	Click IP Configuration. In the Addressing Table above, you can see that the IP address for PC1 is supposed to be 192.168.1.1 and the subnet mask 255.255.255.0. Enter this information for PC1 in the IP Configuration window.

c. 	Repeat steps 1a and 1b for PC2.

Step 2: Test connectivity to switches.

a. 	Click PC1. Close the IP Configuration window if it is still open. In the Desktop tab, click Command Prompt.

b. 	Type the ping command and the IP address for S1, and press Enter.

Packet Tracer PC Command Line 1.0

PC> ping 192.168.1.253

Were you successful? Explain.

____________________________________________________________________________________

Part 3: Configure the Switch Management Interface

Step 1: Configure S1 with an IP address.

Switches can be used without any configurations. Switches forward information from one port to another based on Media Access Control (MAC) addresses. Why does a switch need an IP address?

_______________________________________________________________________________________

_______________________________________________________________________________________

a. 	In the global configuration mode, enter the following commands to configure S1 with an IP address in VLAN 1.

S1# configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)# interface vlan 1

S1(config-if)# ip address 192.168.1.253 255.255.255.0

S1(config-if)# no shutdown

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

S1(config-if)#

S1(config-if)# exit

S1#

What does the no shutdown command?

____________________________________________________________________________________

b. 	Save the configuration.

S1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

S1#

c. 	Verify the IP address configuration on S1.

S1# show ip interface brief

<output omitted>

Vlan1              	192.168.1.253   YES manual up                	up

Step 2: Configure S2 with an IP address.

Use the information in the addressing table repeat the same process in Step 1 to configure S2 with an IP address. Remember to save and verify your configurations.

Step 3: Verify network connectivity.

Network connectivity can be verified using the ping command. It is very important that connectivity exists throughout the network.

a. 	Click PC1, and then click the Desktop tab.

b. 	Open the Command Prompt.

1) 	Ping the IP address for PC2.

2) 	Ping the IP address for S1.

3) 	Ping the IP address for S2.

c. 	From PC2, ping the other devices in the network.

d. 	From S1, ping the other devices in the network. The ping to PC1 is displayed below as an example.

S1> ping 192.168.1.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.1.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 0/0/1 ms

e. 	From S2, ping the other devices in the network.

Đảm bảo rằng tất cả các máy đều có thể ping thành công. Nếu kết quả ping đầu tiên của bạn là 80%, hãy thử lại; nếu đúng nó phải là 100%. Nếu bạn không thể ping bất kỳ thiết bị nào, hãy kiểm tra cấu hình của bạn để tìm lỗi.